TITSEY ESTATE COMPANY

Registered in England No.  221115

DIRECTORS: D A INNES (Chairman), G A INNES

J H INNES, J A N LAING

SALES ENQUIRIES:

Moorhouse Sandpits

Off Westerham Road

Westerham

Kent TN16 2EU

Tel:  01959 563188

Fax: 01959 561835

24th Oct 2023

NATURAL AGGREGATES

HEALTH AND SAFETY DATA SHEET

PRODUCT:			Crushed Rock:  Sand and Gravel

Aggregates are used as basic construction materials and as components of other products, e.g. concrete, asphalt, bricks, blocks and tiles etc. Aggregates are produced from naturally occurring deposits, consisting of combinations of various materials including quartz and silica.

MAIN HAZARD:		Inhalation or Ingestion of dust is harmful, if inhaled in   excessive quantities over a prolonged period or extended period, respirable dust can cause a long term health hazard, but where Crystalline Silica or Quartz are present, this represents an additional hazard.   Crystalline Silica and Quartz make up   approximately 99% of our products.

PRECAUTIONS:		Inhalation or Ingestion of dust should be avoided.   Appropriate dust masks to BS 2091 Type B should be worn when     aggregates containing Quartz or Crystalline Silica are handled or further crushed. The product should be handled to minimise the creation of airborne dust

FIRST AID MEASURSE 	Inhalation

Immediately remove to fresh air. If breathing stopped or irregular, apply artificial respiration and seek medical attention.

Skin Contact

Wash with water. Prolonged contact may cause irritation

Eye Contact

Immediately and thoroughly irrigate with water. The material is abrasive and may scratch surface of eye, if pain persists seek medical attention.

## ACCIDENTAL RELEASE

## MEASURES			Personal Precautions

Avoid breathing in dust

Methods of Cleaning

Spray with water to prevent airborne dust

Avoid dry sweeping, which creates dust.

# EXPOSURE CONTROLS  Take measures to prevent inhalation of dust from aggregates

Spray material with water to avoid airborne dust.

Control limits

Respirable Quartz Crystalline Silica SiO2

Workplace Exposure Limit		0.1mg/m3

Time Weighted Limit			8 hrs

Personal Protection

Suitable dust masks should been worn in enclosed spaces where handling of aggregates is taking place where adequate ventilation is not provided.

To avoid skin contact gloves and overalls to be worn

Goggles may be require for eye protection

TRANSPORTATION:	The carriage of Aggregates is not subject to Hazardous Substance Conveyance Regulations and vehicle labelling is not required.   Aggregates are inert but should be disposed of in accordance with local legal requirements. Open vehicles should be sheeted to avoid dust nuisance.

# ECOLOGICAL INFO	Environmental Assessment

Aggregates are naturally occurring material, when use as intended no adverse effect will occur.

Ecotoxicity

Not expected to be toxic to aquatic organisms

# FIRE FIGHTING MEASURES

Not Applicable

OTHER INFORMATION	 Training advice Wear and use PPE

If you require any further information, please contact the Titsey Estate Company, details as above.